| baremetal ai enhanced security Specification Rev 0.1 | Abstract [Draw your reader in with an engaging abstract. It is typically a short summary of the document. When you’re ready to add your content, just click here and start typing.] Suresh Marisetty CTO |
| --- | --- |

Extending Platform Trust: AI-Augmented Bare Metal Security Across PSA Level 3, Caliptra RoT, CXL, and TDISP

Executive Summary

This specification introduces a modular, AI-enhanced architecture for bare metal security in cloud platforms. Building on PSA Level 3 and Caliptra Root of Trust, it distributes AI agents across Platform RoT, BMC, Host CPU, GPU, and PCIe/CXL devices. These agents collaborate via SPDM, TDISP, and secure telemetry channels to detect anomalies, enforce policies, and extend trust across disaggregated fabrics.

1. Background and Motivation

- PSA Level 3 and Caliptra RoT provide secure boot, attestation, and lifecycle anchors.

- Runtime threats—firmware abuse, side-channel attacks, rogue DMA—require continuous observability.

- AI enables scalable anomaly detection, federated learning, and adaptive trust enforcement.

- CXL and TDISP introduce new trust boundaries across memory and device fabrics.

2. System Components and Roles

| Component | RoT Role | SPDM Role | AI Capability Location |
| --- | --- | --- | --- |
| Platform RoT (PA RoT) | Immutable RoT | SPDM Requester | Central AI orchestrator, policy engine |
| BMC | Extended RoT | SPDM Responder | Firmware integrity, lifecycle monitoring |
| Host CPU | Extended RoT | SPDM Responder | Boot validation, syscall anomaly detection |
| GPU / Accelerators | Extended RoT | SPDM Responder | ECC monitoring, model drift detection |
| PCIe/CXL Devices | Extended RoT | SPDM Responder | DMA audit, config space integrity |

3. AI Distribution and Collaboration Model

- Platform RoT coordinates federated learning and policy enforcement.

- Responders run lightweight AI agents for local anomaly detection.

- SPDM provides attestation and transcript logging.

- Telemetry is streamed via MCTP/PLDM or DOE.

- TDISP enforces secure protocol behavior across PCIe/CXL links.

4. Telemetry Schema per Component

| Component | Key Telemetry Signals |
| --- | --- |
| Platform RoT | SPDM transcripts, trust status, policy logs |
| BMC | Firmware hashes, IPMI logs, sensor anomalies |
| Host CPU | Boot logs, syscall traces, memory access patterns |
| GPU | ECC errors, floating-point anomalies, power traces |
| PCIe/CXL | DMA logs, config snapshots, IDE stream status |

5. Threat Mapping and AI Detection Coverage

| Threat Category | AI Detection Role |
| --- | --- |
| Software Integrity | Boot pattern analysis, firmware hash validation |
| Storage Protection | Access pattern anomaly detection |
| Interface Exploits | Mailbox fuzzing, debug traffic analysis |
| Crypto Weakness | Entropy analysis, impersonation detection |
| Side-Channel Attacks | Timing/power anomaly detection |
| Fault Injection | Voltage/clock glitch signature recognition |

5. Standards Alignment: PSA Level 3, Caliptra RoT, and OCP S.A.F.E

TrustForgeAI’s architecture is designed to align with the most rigorous platform security standards across cloud, embedded, and hyperscale environments. Our modular RoT implementation leverages Caliptra and supports conformance with both PSA Certified Level 3 and OCP S.A.F.E..

🧱 PSA Level 3: Embedded Assurance

PSA Certified Level 3 provides penetration-tested assurance for embedded platforms, covering:

- Secure boot and firmware update flows

- Cryptographic key isolation and attestation

- Threat modeling and lifecycle management

- Compliance with SESIP and GlobalPlatform APIs

TrustForgeAI’s RoT modules are engineered to meet PSA Level 3 requirements, enabling secure provisioning, telemetry, and federated trust across constrained devices.

🏢 OCP S.A.F.E.: Hyperscale Firmware Transparency

OCP S.A.F.E. (Security Appraisal Framework and Enablement) complements PSA by focusing on:

- Firmware and microcode integrity for server-class components

- Secure update mechanisms and rollback protection

- Manual code inspection by approved Security Review Providers (SRPs)

- Open-source alignment and GitHub-based review artifacts

TrustForgeAI’s telemetry and runtime trust orchestration are designed to integrate with OCP S.A.F.E. workflows, enabling hyperscale operators to validate firmware provenance and runtime behavior.

🔗 Caliptra RoT: Silicon-Rooted Trust

Caliptra provides a silicon-based Root of Trust with:

- Secure boot, measurement, and attestation

- Integration with DICE and SPDM protocols

- Compatibility with both PSA and OCP S.A.F.E. assurance models

TrustForgeAI extends Caliptra’s capabilities with modular telemetry hooks and federated learning support, enabling runtime trust across diverse platform segments.

## 🧠 Strategic Convergence

By aligning with both PSA Level 3 and OCP S.A.F.E., TrustForgeAI bridges the assurance gap between embedded and hyperscale platforms. This dual conformance enables:

- Unified trust posture across cloud, edge, and automotive domains

- Reduced audit overhead for hyperscalers and OEMs

- Transparent, standards-driven security for modular RoT deployments

6. Alignment with PSA Level 3 and Caliptra

| PSA/Caliptra Layer | AI Enhancement Role |
| --- | --- |
| Secure Services | Attestation, key management observability |
| Firmware RoT | Boot sequence validation, manifest integrity |
| Hardware RoT | Sensor monitoring, physical tamper detection |
| Lifecycle Anchors | Provisioning, update validation, decommissioning |
| SPDM Protocol | Identity exchange, transcript logging |

7. Integration of CXL and TDISP

| Component | CXL/TDISP Role | AI Role |
| --- | --- | --- |
| Platform RoT (PA RoT) | CXL root complex, TDISP policy enforcer | AI policy hub, federated model coordinator |
| BMC | TDISP-aware provisioning and attestation | Firmware integrity, lifecycle monitoring |
| Host CPU | CXL host interface, memory guard | Boot validation, syscall anomaly detection |
| GPU / CXL Type 1/2 | TDISP endpoint | ECC monitoring, model drift detection |
| Smart Memory / CXL.mem | TDISP endpoint | Version freshness, memory access audit |
| PCIe Switches | IDE stream integrity | DMA audit, replay detection |

8. Modular Code Components for UEFI and SMM

| Module Name | Purpose | Integration Targets |
| --- | --- | --- |
| SPDM Transcript Logger | Capture and verify SPDM exchanges | Platform RoT, UEFI DXE, SMM handler |
| AI Telemetry Collector | Aggregate runtime signals | All RoT components, DXE, SMM |
| Manifest Validator | Validate firmware manifests | Host CPU, GPU, PEI/DXE, SMRAM |
| Lifecycle Anchor Manager | Track provisioning and updates | BMC, Platform RoT, Capsule monitor |
| TDISP Policy Enforcer | Enforce stream isolation and replay guard | PCIe/CXL endpoints, DXE, SMM |
| Boot Sequence Profiler | Analyze boot flow integrity | Host CPU, PEI/DXE, SMM snapshot |
| ECC & Drift Monitor | Detect GPU memory corruption | GPU, DXE, SMM |
| Secure DMA Auditor | Monitor PCIe DMA patterns | PCIe endpoints, DXE, SMM |

9. Additional Modular Enhancements for UEFI and SMM

| Enhancement Module | Purpose |
| --- | --- |
| Runtime Policy Engine | Enforce AI-derived security policies during boot/runtime |
| Secure Event Journal | Append-only log of security events with TPM sealing |
| Entropy Quality Monitor | Validate RNG entropy sources and health |
| Firmware Behavior Profiler | Trace execution paths and detect deviations |
| IDE Stream Validator | Monitor IDE/TDISP stream setup and integrity |
| Update Provenance Verifier | Validate firmware update origin and freshness |
| SMRAM Guard Monitor | Detect unauthorized SMRAM access attempts |
| Caliptra Mailbox Proxy | Expose Caliptra telemetry to UEFI/SMM |
| Federated Learning Agent | Participate in model training during boot |
| TDISP-Aware Device Filter | Enforce device access policies based on TDISP stream status |

10. Implementation Strategy

- Language: C/C++ (EDK2), Assembly/C (SMM)

- Packaging: Modular libraries with defined APIs

- Testing: QEMU/OVMF, Intel SDE, Chipsec

- Security: SMRAM isolation, stack canaries, hardened memory access

11. Standardized Telemetry Schema

Core Object Structure

json

{

"component_id": "host_cpu_01",

"timestamp": "2025-10-28T10:59:00Z",

"firmware_version": "1.2.3",

"boot_phase": "DXE",

"telemetry_type": "runtime",

"trust_state": "attested",

"metrics": {

"ecc_errors": 2,

"dma_anomalies": 0,

"syscall_outliers": 5,

"entropy_quality": 0.92,

"tdisp_stream_status": "valid"

},

"events": [

{

"event_id": "evt_001",

"event_type": "manifest_validation",

"severity": "warning",

"details": "Version freshness check failed"

}

],

"attestation": {

"spdm_transcript_hash": "abc123...",

"tdisp_nonce": "xyz789...",

"pcr_values": {

"0": "hash0",

"1": "hash1"

}

}

}

## 📡 Overview of OpenBMC Telemetry

OpenBMC telemetry is designed to expose platform metrics in a structured, vendor-neutral format. It enables system administrators and orchestration tools to monitor hardware health, performance, and environmental conditions across server platforms.

### 🔧 Core Capabilities

- Sensor Aggregation: Collects and organizes data from onboard sensors (e.g., temperature, voltage, fan speed).

- Metric Reports: Supports on-demand creation of metric reports—grouped sets of sensor values.

- Push and Pull Models:

  - Pull: Clients can query metrics via Redfish API.

  - Push: Metrics can be streamed or triggered based on thresholds or events.

- Redfish TelemetryService Integration: Implements the Redfish TelemetryService schema for standardized access and interoperability.

- Runtime Monitoring: Continuously tracks sensor updates and system state changes.

### 📦 Implementation Details

- Telemetry Middleware: Written in C++ with Meson build system; integrates with OpenBMC’s D-Bus infrastructure.

- Service File: xyz.openbmc_project.Telemetry.service defines the telemetry daemon.

- Build Options:

  - Yocto/Bitbake: For embedded builds.

  - Meson/Ninja: For native development and testing.

### 🔧 Hardware & Sensor Events

- Temperature threshold exceeded

- Voltage out-of-range

- Fan failure or speed anomaly

- Power supply unit (PSU) failure

- DIMM or CPU presence/health change

- NVMe or storage device errors

### ⚡ Power & Boot Events

- Host power on/off

- BMC reboot or crash

- Boot progress codes

- Watchdog timeout

- System reset initiated

### 🔐 Security & Access Events

- Failed login attempts (SSH, Redfish, IPMI)

- User account changes

- Certificate updates or revocation

- SPDM or attestation failures (if enabled)

### 🔄 Firmware & Configuration Events

- BIOS or BMC firmware update

- Configuration changes (e.g., network, hostname)

- Sensor calibration or reconfiguration

- Redfish schema violations or malformed requests

### 🧪 Diagnostic & Debug Events

- Core dumps or crash logs

- System dumps triggered

- Hardware fault isolation (guarded components)

- Callouts for service actions

## OpenBMC Event Table

| Event Code | Event Type | Description |
| --- | --- | --- |
| 0x0001 | System Boot | Host system powered on and boot sequence initiated |
| 0x0002 | System Shutdown | Host system powered off |
| 0x0003 | BMC Reboot | BMC firmware restarted or reset |
| 0x0100 | Sensor Threshold Breach | Sensor reading exceeded defined threshold (e.g., temperature, voltage) |
| 0x0101 | Fan Failure | Fan speed anomaly or complete failure detected |
| 0x0102 | PSU Failure | Power Supply Unit fault or loss of redundancy |
| 0x0200 | DIMM Error | Memory module error or ECC fault |
| 0x0201 | CPU Fault | Processor health degraded or thermal event |
| 0x0300 | Firmware Update Started | BIOS or BMC firmware update initiated |
| 0x0301 | Firmware Update Completed | Firmware update completed successfully |
| 0x0302 | Firmware Update Failed | Firmware update failed or was aborted |
| 0x0400 | Watchdog Timeout | System watchdog expired, indicating hang or crash |
| 0x0500 | User Login Failed | Failed login attempt via SSH, IPMI, or Redfish |
| 0x0501 | User Account Modified | User account created, deleted, or modified |
| 0x0600 | Certificate Updated | TLS or Redfish certificate replaced or revoked |
| 0x0700 | SPDM Attestation Failure | Secure protocol attestation failed or was rejected |
| 0x0800 | Crash Dump Triggered | System or BMC crash dump captured |
| 0x0900 | Component Guarded | Faulty hardware component marked as guarded (e.g., DIMM, CPU) |
| 0x0A00 | Redfish Request Error | Malformed or unauthorized Redfish API request |
| 0x0B00 | Configuration Changed | Network, hostname, or other system config updated |

## Standard UEFI BIOS Event Log Flow to BMC

### 🧩 Typical Event Sources in UEFI

- POST codes and boot progress

- Firmware update status

- Hardware initialization errors (e.g., DIMM, CPU, PCIe)

- Secure boot violations

- Platform configuration changes

| Event Code | Event Name | Description |
| --- | --- | --- |
| 0x3000 | BIOS Boot Initiated | BIOS has started the boot process |
| 0x3001 | POST Completed | Power-On Self-Test completed successfully |
| 0x3002 | Boot Device Selected | BIOS has selected the boot device |
| 0x3003 | Secure Boot Violation | Secure Boot failed due to unauthorized firmware or configuration |
| 0x3004 | Firmware Update Started | BIOS firmware update initiated |
| 0x3005 | Firmware Update Completed | BIOS firmware update completed successfully |
| 0x3006 | Firmware Update Failed | BIOS firmware update failed or was aborted |
| 0x3007 | CMOS Battery Low | BIOS detected low voltage on CMOS battery |
| 0x3008 | BIOS Configuration Changed | BIOS settings modified (e.g., boot order, virtualization flags) |
| 0x3009 | BIOS Password Changed | Setup password was updated via BMC or host interface |
| 0x300A | Hardware Initialization Error | BIOS detected fault during DIMM, CPU, or PCIe initialization |
| 0x300B | BIOS Recovery Triggered | Recovery BIOS image loaded due to corruption or failure |
| 0x300C | BIOS Guard Activated | BIOS Guard or similar protection mechanism triggered |
| 0x300D | BIOS Version Mismatch | BIOS version does not match expected platform configuration |

12.1 OpenBMC Telemetry Extensions for for Baremetal AI Platforms

To support federated AI platforms anchored in Caliptra RoT and PSA Certified trust models, OpenBMC can be extended to emit AI-aware events that enhance observability, attestation, and runtime assurance. These events help bridge firmware, telemetry, and orchestration layers in distributed environments like those envisioned by TrustForgeAI

|  | Description |
| --- | --- |
| spdmTranscriptHash | SPDM attestation context |
| tdispStreamStatus | TDISP stream validity |
| anomalyScore | AI model output |
| featureVector | Input features for anomaly detection |
| policyDecision | Action taken based on AI inference |

| Event Name | Description | Trigger Point |
| --- | --- | --- |
| FederatedNode_Provisioned | Node successfully onboarded with federated identity and trust anchor | After SPDM attestation and identity binding |
| AI_TelemetryAgent_Activated | Secure telemetry agent initialized and linked to RoT | During OS boot or BMC handshake |
| Caliptra_Attestation_Completed | Caliptra RoT attestation completed and verified | Post-boot, during SPDM/EAT exchange |
| AI_Model_Integrity_Verified | Hash/signature of deployed AI model verified against expected value | Before model execution |
| SecureInference_Engine_Initialized | Hardware accelerator (GPU/NPU) initialized with RoT-bound telemetry | During device enumeration or OS boot |
| FederatedTelemetry_Stream_Started | Telemetry stream securely routed to federated trust aggregator | Post-boot, after network provisioning |
| AI_Workload_Quarantined | Workload isolated due to failed attestation, anomaly, or policy violation | Runtime monitoring or BMC policy enforcement |
| RoT_Health_Degraded | Caliptra or SPDM trust anchor reported degraded status | During periodic health check or telemetry scan |
| SecureMemory_Region_Allocated | Memory region allocated for AI model or telemetry with RoT protection | During firmware handoff to OS |
| FederatedPolicy_UpdateReceived | Updated trust or telemetry policy received from federated controller |  |

Benefits for Federated AI:

These events enable secure, standards-aligned observability across federated AI nodes, supporting runtime trust, policy enforcement, and model integrity verification. Let me know if you'd like help drafting Redfish schema extensions or visualizing this event flow across Caliptra, SPDM, and OpenBMC.

12.2 Suggested UEFI Events for Baremetal AI Platforms

To support Baremetal AI enhancements—especially in federated, RoT-anchored environments like those envisioned by TrustForgeAI—UEFI firmware can be extended to emit AI-relevant events that improve observability, attestation, and runtime assurance. These events would bridge the gap between low-level platform state and high-level AI orchestration.

| Event Name | Purpose | Trigger Point |
| --- | --- | --- |
| AI_Workload_Intent_Detected | Signals that the OS or hypervisor is preparing to launch an AI workload | During boot or handoff to OS |
| RoT_Attestation_Completed | Confirms successful attestation of platform RoT (e.g., Caliptra) | After SPDM/EAT token exchange |
| Telemetry_Agent_Provisioned | Indicates that AI telemetry agent has been securely provisioned | Post-boot, during BMC handshake |
| Federated_Node_Identity_Bound | Logs binding of federated identity to platform (e.g., via DICE or TPM) | During secure boot or attestation phase |
| AI_Model_Integrity_Verified | Confirms hash/signature of deployed AI model matches expected value | Before model execution |
| Runtime_Trust_Anchor_Activated | Indicates activation of runtime trust anchor for AI telemetry | During OS boot or agent launch |
| Secure_Memory_Region_Allocated | Logs allocation of memory for AI model or telemetry with RoT protection | During firmware handoff to OS |
| Inference_Engine_Initialized | Signals that hardware acceleration (e.g., GPU/NPU) is initialized securely | During device enumeration or OS boot |
| Federated_Telemetry_Stream_Started | Marks the start of telemetry stream to federated trust aggregator | Post-boot, after network provisioning |
| AI_Workload_Quarantined | Logs when a workload is isolated due to failed attestation or anom |  |

12.3 Suggested RoT Event Log Extension to BareMetal AI

To support Federated AI platforms anchored in Caliptra-based Root of Trust (RoT), OpenBMC can be extended to emit and manage Caliptra-specific events that enhance attestation, telemetry, and runtime trust. These events bridge secure firmware with orchestration layers, enabling scalable observability and policy enforcement across distributed AI nodes.

| Event Name | Description | Trigger Point |
| --- | --- | --- |
| Caliptra_Provisioning_Started | Indicates the start of Caliptra RoT provisioning and key injection | During manufacturing or secure onboarding |
| Caliptra_Provisioning_Completed | Confirms successful provisioning of Caliptra RoT | Post key injection and identity binding |
| Caliptra_Attestation_Requested | BMC or host has initiated attestation via SPDM or EAT | During boot or telemetry handshake |
| Caliptra_Attestation_Successful | Attestation completed and verified by BMC or remote verifier | After SPDM/EAT token validation |
| Caliptra_Attestation_Failed | Attestation failed due to integrity mismatch or expired credentials | During boot or runtime trust check |
| Caliptra_Firmware_Measurement | Logs firmware hash or measurement extended into PCR or digest | During secure boot or firmware update |
| Caliptra_Telemetry_Anchor_Activated | Secure telemetry anchor initialized for runtime monitoring | Post-attestation, before workload launch |
| Caliptra_Health_Degraded | RoT health check failed or reported degraded status | Periodic polling or triggered by anomaly |
| Caliptra_Recovery_Triggered | Recovery flow initiated due to RoT corruption or failed attestation | During boot or runtime monitoring |
| Caliptra_Ephemeral_ID_Refreshed | Ephemeral identity or key refreshed for federated trust rotation |  |

## Benefits for Federated AI

- Enables secure onboarding and runtime trust for distributed AI workloads

- Supports policy enforcement and anomaly detection via RoT telemetry

- Facilitates lifecycle management of RoT components across heterogeneous platforms

## 🧩 Drafting Event Structures for UEFI, OpenBMC & RoT

### 1. UEFI Event Structure (SMBIOS Type 15 Extension)

SMBIOS Type 15 defines the Event Log structure. For federated AI, you can extend it with:

- Event Type: AI workload intent, RoT attestation, telemetry agent provisioning

- Event Data Format: Include SHA-256 digests, EAT tokens, or UUIDs for federated identity

- Trigger Source: Firmware module, RoT, or OS handoff

- Severity: Info, Warning, Critical

- Timestamp: UTC with monotonic boot time reference

Example:

plaintext

Event Type: 0xF1 (Federated Identity Bound)

Event Data: UUID + RoT digest

Severity: Info

Timestamp: 2025-11-04T09:51:00Z

### 2. OpenBMC Event Structure (D-Bus & Redfish LogServices)

Extend xyz.openbmc_project.Logging.Entry and Redfish /LogServices/EventLog/Entries with:

- EventId: Unique code (e.g., 0xFA01 for Caliptra attestation success)

- Message: Human-readable summary

- SensorType: RoT, AI Agent, Secure Memory

- EntryType: Event, Alert, Recovery

- AdditionalDataURI: Link to telemetry snapshot or attestation report

Example Redfish Entry:

json

{

"EventId": "FA01",

"Message": "Caliptra attestation completed successfully",

"Severity": "OK",

"SensorType": "RootOfTrust",

"EntryType": "Event",

"Created": "2025-11-04T09:51:00Z"

}

### 3. RoT Event Structure (Caliptra + SPDM/EAT)

Caliptra firmware can emit structured events via SPDM or PLDM:

- Event Class: Attestation, Identity, Integrity, Recovery

- Digest: SHA-256 of firmware or telemetry agent

- EAT Token: Encoded evidence of platform state

- Nonce: For freshness

- Verification Status: Passed/Failed

Example SPDM Extension:

json

{

"EventClass": "Attestation",

"Digest": "abc123...",

"EAT": "<token>",

"Nonce": "xyz789",

"Status": "Verified"

}

13. Visual Schema Flow

## Redfish Schema Extension for RoT Events

To log Caliptra RoT events in OpenBMC and expose them via Redfish, we extend the LogEntry schema under LogServices/EventLog/Entries.

### 🔧 Extended Redfish LogEntry Fields

json

{

"@odata.type": "#LogEntry.v1_10_0.LogEntry",

"Id": "123",

"EventId": "FA01",

"Message": "Caliptra attestation completed successfully",

"Severity": "OK",

"SensorType": "RootOfTrust",

"EntryType": "Event",

"EventTime": "2025-11-04T09:51:00Z",

"AdditionalDataURI": "/redfish/v1/TelemetryService/Reports/CaliptraDigest123",

"Oem": {

"TrustForgeAI": {

"RoTDigest": "abc123...",

"EATToken": "eyJhbGciOiJFUzI1NiIsInR5cCI6IkVhdCJ9...",

"Nonce": "xyz789",

"VerificationStatus": "Verified"

}

}

}

### 🧩 Key Additions

- SensorType: RootOfTrust — tags RoT-originated events

- AdditionalDataURI — links to telemetry or attestation report

- Oem.TrustForgeAI — OEM extension for federated trust metadata

This structure supports Caliptra attestation, digest verification, and telemetry anchoring in federated AI environments.

## 🧠 Visual Integration Flow

The diagram above shows how RoT events flow from Caliptra → OpenBMC → Redfish → Federated Trust Controller:

- Caliptra emits ephemeral events (attestation, digest, identity)

- OpenBMC logs events via D-Bus and Redfish, tagging them with RoT metadata

- Federated Trust Controller ingests logs, verifies EAT tokens, and activates telemetry stream

This enables secure, standards-aligned observability for AI workloads across distributed nodes.

Events Storage:

| Event Type | Generated By | Stored In | Notes |
| --- | --- | --- | --- |
| Firmware measurements | Caliptra | Passed to BMC or host via SPDM | Not stored locally in Caliptra |
| Attestation success/failure | BMC | Logged via D-Bus or Redfish | Based on SPDM/EAT verification |
| Identity binding / provisioning | BMC | Logged in OpenBMC event log | May include UUID, nonce, digest |
| RoT health status | Caliptra → BMC | Logged periodically or on fault | BMC polls or receives status |
| Ephemeral key rotation | Caliptra | Logged by BMC if policy requires | Useful for federated trust rotation |

Component-Level Flow:

- Host CPU, GPU, Smart Memory → BMC, PCIe/CXL → Platform RoT → AI Orchestrator

Stages:

- Local collection

- Secure transport

- Aggregation

- AI analysis

- Policy enforcement

15. Hardware Requirements

🧠 AI Model Requirements for Federated Learning with RoT

Federated AI typically avoids large centralized models like GPT-style LLMs. Instead, it uses:

✅ Model Types

- Lightweight neural networks (e.g., CNNs, RNNs, small transformers)

- Quantized models for edge inference (e.g., MobileNet, TinyML)

- Split learning or vertical federated models for privacy-preserving collaboration

- Confidential federated learning (CFL) models that run inside secure enclaves

These models are trained across distributed nodes and aggregated securely, often using differential privacy or homomorphic encryption.

15.1 🔐 CPU and RoT Requirements

To support PSA Certified Level 2 or 3 and federated AI, the CPU inside the RoT must offer:

✅ Trusted Execution Environment (TEE)

- Arm TrustZone (for Cortex-A or Cortex-M) or equivalent

- Intel SGX, AMD SEV, or RISC-V PMP for secure memory isolation

- Microsoft Pluton for integrated TPM and firmware update resilience

✅ Cryptographic Acceleration

- Hardware support for AES, SHA, ECC, RSA, and optionally PQC

- Enables secure model aggregation, telemetry signing, and attestation

✅ Secure Boot and Firmware Update

- ROM-based bootloader with signature verification

- PSA Firmware Update API compliance

- Rollback protection and version control

✅ Attestation Engine

- Support for Entity Attestation Token (EAT) or CBOR Web Token (CWT)

- Enables remote trust validation of model execution and telemetry

✅ Secure Storage

- Isolated flash/NVM for model weights, credentials, and logs

- PSA Secure Storage API compliance

🧱 Example CPU Architectures

| Architecture | RoT Feature Support | Use Case |
| --- | --- | --- |
| Arm Cortex-M with TrustZone | PSA Level 2/3, Secure Storage, Crypto | IoT, edge nodes |
| Intel SGX | Enclave-based model execution | Cloud aggregation |
| AMD SEV | VM-level isolation | Federated training in data centers |
| RISC-V with PMP + Caliptra | Modular RoT, open-source attestation | Custom silicon, automotive |
| Microsoft Pluton | TPM 2.0, firmware resilience | Windows endpoints, AI PCs |

16. Conclusion and Next Steps

This architecture bridges static RoT enforcement with dynamic, AI-driven runtime security. By embedding AI across bare metal components and coordinating via SPDM, TDISP, and telemetry, cloud platforms can achieve scalable, adaptive trust.

Next Steps:

- Develop proof-of-concept modules

- Standardize telemetry schemas

- Publish reference implementations

- Engage ecosystem partners for validation

Telemetry-Diagram